ASProNet: Abnormal Situation Prognosis Network for Complex Chemical Processes

Zhiyuan Xie1, 2, Borui Yang1, 2, Jinghe Zheng4, Shui Qin4, Yuhe Tian5, Jinsong Zhao1, 2, 3, *

¹Department of Chemical Engineering, Tsinghua University, Beijing, 100084, China

2State Key Laboratory of Chemical Engineering and Low-carbon Technology, Beijing 100084, China

3School of Education, Tsinghua University, Beijing 100084, China

4Sinopec Jiujiang Company, Jiujiang 332004, China

5Department of Chemical and Biomedical Engineering, West Virginia University, Morgantown, WV 26506, USA

Corresponding author. Tel: 010-62783109; E-mail: jinsongzhao@tsinghua.edu.cn

Abstract

Abnormal situation prognosis is indispensable for maintaining stable and safe operation in industrial chemical processes. However, abnormal situations occur rarely in industrial systems, leading to strongly imbalanced operating data and making early identification of precursor behaviors difficult. To address these formidable challenges, we present an Abnormal Situation Prognosis Network (ASProNet), a Transformer-based model that integrates a dual-resolution temporal encoder with progressive steady-transient decomposition. The ASProNet incorporates a gated mixture-of-experts predictor featuring per-horizon blending and a tail-aware training strategy, specifically designed to amplify predictive sensitivity near alarm thresholds. We validate the ASProNet using six months of continuous operational data from an industrial fluid catalytic cracking (FCC) unit with an annual capacity of 1 Mt·yr⁻¹. The ASProNet’s performance is evaluated across three critical prognosis tasks: stabilizer overhead pressure, regenerator pressure, and the reactor–regenerator pressure difference. At a 15-minute prognosis horizon, the ASProNet reaches a prognostic recall rate of 91.9%, 97.5% and 96.0% on the three tasks. Its false prognosis rate stays between 0.13% and 0.34%, or 0.013–0.034 false alarms per 10 min. The mean prognosis lead times are 12.1, 10.2, and 11.6 min, respectively, exceeding the best baseline by an average of 6.0 min across the three tasks. These results demonstrate a potential paradigm shift in abnormal situation management (ASM), moving from reactive response toward proactive prognosis with high reliability in large-scale industrial chemical plants.

Abnormal Situation Management (ASM) is an indispensable core component for ensuring process safety in chemical operations [1–3]. According to the leading and lagging metrics of the AIChE Center for Chemical Process Safety (CCPS), almost every major incident is caused by an abnormal situation [4]. These major accidents pose severe threats to human safety, economic stability, and environmental sustainability [5–12]. Over the past two decades, chemical process accidents in China have led to 4,088 fatalities, with the worst single accident claiming 62 lives and causing USD 100 million in direct losses [13–15]. In addition to triggering major accidents, abnormal situations cause minor incidents worldwide [1], such as off-specification products, unplanned shutdowns, equipment damage, and environmental emission violations [16–18]. Statistics indicate that abnormal situations cost the petroleum and chemical industries in the United States 20 billion US dollars per year and those in the United Kingdom 27 billion US dollars per year [12,19]. The cost is much more when similar situations in other industries such as pharmaceutical, desalination, power, specialty chemicals etc. are included [1].

To manage abnormal situations, industrial practice has long relied on fault detection and diagnosis (FDD) as a core analytical component of ASM [2,16,20]. In chemical processes, FDD is primarily concerned with identifying whether the current process state has departed from normal operation and, if so, determining the likely cause or fault class responsible for that deviation [20,21]. This capability is indispensable for process monitoring, but its decision point is typically tied to abnormalities that have already become observable in the measured process state [20,22]. By this stage, these abnormalities often generate alarm floods that overwhelm operators, heighten cognitive load, and delay critical interventions [23,24]. As a result, although FDD can support timely response once a fault is detected, the prevailing ASM workflow remains largely reactive, in the sense that corrective action is usually initiated only after the process has entered an abnormal situation [3,16,20]. In contrast, abnormal situation prognosis predicts future trends of process variables to determine whether an abnormal situation is developing before an alarm triggers [22,25–27]. In this sense, abnormal situation prognosis can be regarded as a forward extension of the conventional FDD framework, shifting ASM from response after fault occurrence toward proactive intervention during the early evolution of abnormal situations [22,25,28,29].

The core target is to reliably prognose these abnormal situations, securing a critical intervention window before abnormal situations fully manifest [3,16,22,28]. In industrial chemical processes, however, process data are overwhelmingly dominated by normal operating situations, whereas observations capturing these abnormal situations are exceedingly scarce [22,23,30]. Despite this imbalance, most existing studies treat prognosis as a conventional time-series forecasting problem and train models by minimizing global prediction error [22,31,32]. It inherently assumes equal operational significance for normal and abnormal situations. Yet, in industrial chemical processes where normal data overwhelmingly dominate, conventional model becomes intrinsically biased toward the densely sampled normal regime. Consequently, such conventional abnormal situation prognosis approaches frequently fail to deliver reliable abnormal situation prognosis [30–32], revealing a structural mismatch between current prognosis paradigms and practical industrial prognosis demands.

Addressing this mismatch requires an abnormal situation prognosis model that can represent temporal dependencies in chemical process data, while remaining sensitive to operating situations [22,33,34]. Transformer-based models provide a natural starting point for this task, and they have shown strong performance in recent studies on chemical process fault detection, diagnosis and prognosis [22,35,36]. However, a single forecasting model is still liable to devote most of its representational capacity to the dominant patterns of normal situation [32,33,37]. Mixture-of-experts models offer a principled way to address this limitation by allowing different experts to specialize in different data regimes under the coordination of a gating mechanism [38–40]. For abnormal situation prognosis, this architecture is especially attractive because the process evolution near alarm thresholds need not follow the same predictive regularities as those governing normal situation. Motivated by this, we develop a mixture-of-experts transformer for abnormal situation prognosis (ASProNet) in chemical processes. The model extracts multi-scale temporal representations through dual-resolution patch encoding with learnable recency priors and dense cross-attention to exogenous variables. Within the encoder, progressive series decomposition separates transient and steady components, which are then forecast by specialized prediction experts and blended through a per-horizon gating mechanism. To further improve the relevance of training to industrial abnormal situation prognosis, samples closer to alarm thresholds are assigned greater influence during optimization. In this way, the proposed model is designed not merely to improve average forecasting accuracy in normal situation, but to improve prediction where abnormal situation prognosis is most operationally valuable: during the early stage of abnormal situation.

Acknowledgement

This work was supported by the National Key Research and Development Program of China (2024YFC3013601).

References

[1]	F. Eljack, M.-K. Kazi, Process safety and abnormal situation management, Curr. Opin. Chem. Eng. 14 (2016) 35–41. https://doi.org/10.1016/j.coche.2016.07.004.

[2]	S. Liu, F. Lei, D. Zhao, Q. Liu, Abnormal Situation Management in Chemical Processes: Recent Research Progress and Future Prospects, Processes 11 (2023) 1608.

[3]	Y. Dai, H. Wang, F. Khan, J. Zhao, Abnormal situation management for smart chemical process operation, Curr. Opin. Chem. Eng. 14 (2016) 49–55. https://doi.org/10.1016/j.coche.2016.07.009.

[4]	S. Kenan, S. Kadri, Process safety leading indicators survey–February 2013: Center for chemical process safety–white paper, Process Saf. Prog. 33 (2014) 247–258. https://doi.org/10.1002/prs.11654.

[5]	F. Lees, Lees’ Loss prevention in the process industries: Hazard identification, assessment and control, Butterworth-Heinemann, 2012.

[6]	U.S. CHEMICAL SAFETY AND HAZARD INVESTIGATION BOARD, Investigation Report: Refinery Explosion and Fire (March 23, 2005, Texas City, Texas), U.S. Chemical Safety Board, 2007. https://www.csb.gov/file.aspx?DocumentId=5596.

[7]	UNEP, Global chemicals outlook II: From legacies to innovative solutions, UNEP Geneva, Switzerland, 2019.

[8]	Marsh, The 100 Largest Losses 1974-2013: Large property damage losses in the Hydrocarbon Industry, Marsh, 2014.

[9]	B.K. Kim, J. Krams, E. Krug, M. Leaseburge, J. Lemley, A. Alkhawaldeh, R.A. Mentzer, M.S. Mannan, Case study analysis of the financial impact of catastrophic safety events, J. Loss Prev. Process Ind. 25 (2012) 780–787.

[10]	State Council Investigation Team of the Jiangsu Xiangshui “3·21” Particularly Major Explosion Accident, Investigation Report of the “3·21” Particularly Major Explosion Accident at Jiangsu Xiangshui Tenjia Yi Chemical Co., Ltd., 2019. https://www.mem.gov.cn/gk/sgcc/tbzdsgdcbg/2019tbzdsgcc/201911/P020191115565111829069.pdf.

[11]	Ministry of Emergency Management of the People’s Republic of China, Investigation Report on the Tianjin Port “8·12” Ruihai Company Hazardous Materials Fire and Explosion Accident, Ministry of Emergency Management of the People’s Republic of China, 2016. https://www.mem.gov.cn/gk/sgcc/tbzdsgdcbg/2016/201602/P020190415543917598002.pdf.

[12]	I. Nimmo, Adequately address abnormal operations, Chem. Eng. Prog. 91 (1995).

[13]	Y. Zhu, X. Qian, Z. Liu, P. Huang, M. Yuan, Analysis and assessment of the Qingdao crude oil vapor explosion accident: Lessons learnt, J. Loss Prev. Process Ind. 33 (2015) 289–303.

[14]	Y. Gong, Y. Li, STAMP-based causal analysis of China-Donghuang oil transportation pipeline leakage and explosion accident, J. Loss Prev. Process Ind. 56 (2018) 402–413.

[15]	J. Xing, H. Meng, X. Meng, An urban pipeline accident model based on system engineering and game theory, J. Loss Prev. Process Ind. 64 (2020) 104062.

[16]	Y. Shu, L. Ming, F. Cheng, Z. Zhang, J. Zhao, Abnormal situation management: Challenges and opportunities in the big data era, Comput. Chem. Eng. 91 (2016) 104–113. https://doi.org/10.1016/j.compchemeng.2016.04.011.

[17]	H. Vedam, S. Dash, V. Venkatasubramanian, An intelligent operator decision support system for abnormal situation management, Comput. Chem. Eng. 23 (1999) S577–S580. https://doi.org/10.1016/S0098-1354(99)80142-2.

[18]	M.-K. Kazi, F. Mohammed, A.M.N. AlNouss, F. Eljack, Multi-objective optimization methodology to size cogeneration systems for managing flares from uncertain sources during abnormal process operations, Comput. Chem. Eng. 76 (2015) 76–86. https://doi.org/10.1016/j.compchemeng.2015.02.012.

[19]	V. Venkatasubramanian, Prognostic and diagnostic monitoring of complex systems for product lifecycle management: Challenges and opportunities, Comput. Chem. Eng. 29 (2005) 1253–1263. https://doi.org/10.1016/j.compchemeng.2005.02.026.

[20]	V. Venkatasubramanian, R. Rengaswamy, K. Yin, S.N. Kavuri, A review of process fault detection and diagnosis, Comput. Chem. Eng. 27 (2003) 293–311. https://doi.org/10.1016/S0098-1354(02)00160-6.

[21]	N. Md Nor, C.R. Che Hassan, M.A. Hussain, A review of data-driven fault detection and diagnosis methods: applications in chemical process systems, Rev. Chem. Eng. 36 (2020) 513–553. https://doi.org/10.1515/revce-2017-0069.

[22]	Y. Bai, J. Zhao, A novel transformer-based multi-variable multi-step prediction method for chemical process fault prognosis, Process Saf. Environ. Prot. 169 (2023) 937–947. https://doi.org/10.1016/j.psep.2022.11.062.

[23]	P. Goel, A. Datta, M.S. Mannan, Industrial alarm systems: Challenges and opportunities, J. Loss Prev. Process Ind. 50 (2017) 23–36. https://doi.org/10.1016/j.jlp.2017.09.001.

[24]	F.E. Mustafa, I. Ahmed, A. Basit, U.-E.-H. Alvi, S.H. Malik, A. Mahmood, P.R. Ali, A review on effective alarm management systems for industrial process control: Barriers and opportunities, Int. J. Crit. Infrastruct. Prot. 41 (2023) 100599. https://doi.org/10.1016/j.ijcip.2023.100599.

[25]	Y. Bai, S. Xiang, F. Cheng, J. Zhao, A dynamic-inner LSTM prediction method for key alarm variables forecasting in chemical process, Chin. J. Chem. Eng. (2022). https://doi.org/10.1016/j.cjche.2022.08.024.

[26]	R. Arunthavanathan, F. Khan, S. Ahmed, S. Imtiaz, A deep learning model for process fault prognosis, Process Saf. Environ. Prot. 154 (2021) 467–479. https://doi.org/10.1016/j.psep.2021.08.022.

[27]	Y. Bai, H. Ye, J. Zhao, EPMITS: An efficient prediction method incorporating trends and shapes features for chemical process variables, Comput. Chem. Eng. 191 (2024) 108855. https://doi.org/10.1016/j.compchemeng.2024.108855.

[28]	B. Yang, Y. Bai, F. Lv, J. Zheng, S. Qin, J. Zhao, A process pretrained transformer model for cross-unit fault prognosis in chemical process, Process Saf. Environ. Prot. 208 (2026) 108203. https://doi.org/10.1016/j.psep.2025.108203.

[29]	R. Arunthavanathan, F. Khan, S. Ahmed, S. Imtiaz, An analysis of process fault diagnosis methods from safety perspectives, Comput. Chem. Eng. 145 (2021) 107197.

[30]	J. Jiang, C. Zhang, L. Ke, N. Hayes, Y. Zhu, H. Qiu, B. Zhang, T. Zhou, G.-W. Wei, A review of machine learning methods for imbalanced data challenges in chemistry, Chem. Sci. 16 (2025) 7637–7658. https://doi.org/10.1039/D5SC00270B.

[31]	B. Lim, S. Zohren, Time-series forecasting with deep learning: a survey, Philos. Trans. R. Soc. Math. Phys. Eng. Sci. 379 (2021).

[32]	Y. Wang, H. Wu, J. Dong, Y. Liu, C. Wang, M. Long, J. Wang, Deep time series models: A comprehensive survey and benchmark, ArXiv Prepr. ArXiv240713278 (2024).

[33]	R. Ni, Z. Lin, S. Wang, G. Fanti, Mixture-of-linear-experts for long-term time series forecasting, in: Int. Conf. Artif. Intell. Stat., PMLR, 2024: pp. 4672–4680.

[34]	Z. Ke, H. Duan, S. Qian, Interpretable mixture of experts for time series prediction under recurrent and non-recurrent conditions, ArXiv Prepr. ArXiv240903282 (2024).

[35]	X. Bi, J. Zhao, A novel orthogonal self-attentive variational autoencoder method for interpretable chemical process fault detection and identification, Process Saf. Environ. Prot. 156 (2021) 581–597. https://doi.org/10.1016/j.psep.2021.10.036.

[36]	H. Wu, J. Zhao, Deep convolutional neural network model based chemical process fault diagnosis, Comput. Chem. Eng. 115 (2018) 185–197. https://doi.org/10.1016/j.compchemeng.2018.04.009.

[37]	K. Ghosh, C. Bellinger, R. Corizzo, P. Branco, B. Krawczyk, N. Japkowicz, The class imbalance problem in deep learning, Mach. Learn. 113 (2024) 4845–4901.

[38]	R.A. Jacobs, M.I. Jordan, S.J. Nowlan, G.E. Hinton, Adaptive mixtures of local experts, Neural Comput. 3 (1991) 79–87.

[39]	X. Liu, J. Liu, G. Woo, T. Aksu, Y. Liang, R. Zimmermann, C. Liu, S. Savarese, C. Xiong, D. Sahoo, Moirai-moe: Empowering time series foundation models with sparse mixture of experts, ArXiv Prepr. ArXiv241010469 (2024).

[40]	X. Shi, S. Wang, Y. Nie, D. Li, Z. Ye, Q. Wen, M. Jin, Time-moe: Billion-scale time series foundation models with mixture of experts, ArXiv Prepr. ArXiv240916040 (2024).

[41]	J. Zhou, X. Li, D. Liu, F. Wang, T. Zhang, M. Ye, Z. Liu, A hybrid spatial-temporal deep learning prediction model of industrial methanol-to-olefins process, Front. Chem. Sci. Eng. 18 (2024) 42. https://doi.org/10.1007/s11705-024-2403-7.

[42]	J. Dong, X. Liu, R. Su, H. Xu, T. Yu, TCN-Transformer Deep Network with Random Forest for Prediction of the Chemical Synthetic Ammonia Process, ACS Omega 10 (2025) 2269–2279. https://doi.org/10.1021/acsomega.4c09634.

[43]	EEMUA, Alarm Systems: A Guide to Design, Management and Procurement, 4th ed., Engineering Equipment and Materials Users Association, London, UK, 2024.

[44]	Y. Wang, H. Wu, J. Dong, G. Qin, H. Zhang, Y. Liu, Y. Qiu, J. Wang, M. Long, TimeXer: Empowering Transformers for Time Series Forecasting with Exogenous Variables, (2024). https://doi.org/10.48550/arXiv.2402.19072.

[45]	T. Kim, J. Kim, Y. Tae, C. Park, J.-H. Choi, J. Choo, Reversible instance normalization for accurate time-series forecasting against distribution shift, in: Int. Conf. Learn. Represent., 2021.

[46]	A. Vaswani, N. Shazeer, N. Parmar, J. Uszkoreit, L. Jones, A.N. Gomez, Ł. Kaiser, I. Polosukhin, Attention is all you need, Adv. Neural Inf. Process. Syst. 30 (2017).

[47]	Y. Liu, H. Wu, J. Wang, M. Long, Non-stationary transformers: Exploring the stationarity in time series forecasting, Adv. Neural Inf. Process. Syst. 35 (2022) 9881–9893.

[48]	H. Wu, J. Xu, J. Wang, M. Long, Autoformer: Decomposition transformers with auto-correlation for long-term series forecasting, Adv. Neural Inf. Process. Syst. 34 (2021) 22419–22430.